How to Install CH340G Driver

The USB to serial port chip of this control board is CH340G. So you need to install the driver for the chip. You can click the driver file here.

In different systems, the driver installation is similar. Here we start to install the driver on the Win7 system.You can find the “USB_Drive_CH341_3_1” folder in the information we provide, this is the driver file we want to install.

Plug one end of your USB cable into the NANO ch340 board and the other into a USB socket on your computer.

When you connect the Nano/UNO CH340 board to your computer at the first time, right click your “My Computer”—>for “Properties”—>click the “Device manager”, under Other devices, you should see the “USB-Serial”or “Unknown device ”.Or you can search for "devi" in your computer, or you can open the device manager of your computer.

Then right-click on the device and select the top menu option (Update Driver Software...) shown as the figure below.

Then it will be prompted to either “Search Automatically forupdated driversoftware” or “Browse my computer for driver software”. Shown as below. In this page, select “Browse my computer for driver software”.

After that, select the browse option and navigate to the drive folder "USB_Drive_CH341_3_1", which can be found in the information we provide.

Once the software has been installed, you will get a confirmation message. Installation completed, click “Close”.

Up to now, the driver is installed well. Then you can right click “My Computer”—>for “Properties”—>click the “Device manager”, you should see the device as the figure shown below. Or you can search for "devi" in your computer, or you can open the device manager of your computer.